Tail Suspension Test Standard Operating Protocol

Zengyou Ye at Ikemoto Lab, NIDA

- Open Bonsai:

  - Select the appropriate program: ZY_tailSuspension_V8_Opstim.

- Confirm Hardware Selections in Bonsai:

  - Verify the Arduino Uno port, camera, and speaker settings on the right panel.

- Optional - Optogenetic Stimulation Setup:

  - Power on the Doric "Laser Diode Fiber Light Source" and launch the corresponding software.

  - Within the software, connect the "Laser Diode Fiber Light Source".

  - Load an existing configuration or create a new one (gate control with restart option is recommended).

  - Confirm the current output and start the setup, then leave it running.

- Run Bonsai File:

  - This will initiate white noise playback and display a User Interface (UI). If the UI does not appear automatically, click the "Click for UI" node.

- Set Parameters in UI:

  - Enter the file path, User ID, Experimental ID, and Mouse ID in the UI.

- Start the Test:

  - Click the "Start" button.

  - Place the mice onto the stage of the Tail Suspension Test (TST) box.

  - For Optogenetic Stimulation: 
Place the mice first, connect the fiber patch cable to the cannulas, then click "Start"

- Begin Recording:

  - Remove the stage from the TST box to initiate recording and monitor the timer on the UI.

- End Recording:

  - After 6 minutes, replace the stage in the TST box and click the "Stop" button.

- Remove Mice:

  - Take the mice off the stage.

- Repeat for Additional Mice:

  - Repeat steps 5-9 for the next set of mice.